Part Number:	PTMZD-1M160-02

Kit Contents

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 | 1 | Kit Bracket |
| 2 | 1 | Armrest, Main |
| 3 | 1 | Lower Base |
| 4 | 2 | Bushing, Outer LH & RH |
| 5 | 2 | Bushing, Inner LH & RH |
| 6 | 1 | Adapter, Upper |
| 7 | 4 | 6mm Flat Head Screws |
| 8 | 2 | Cap, Side Right & Left |
| 9 | 1 | Bolt, Carriage |
| 10 | 1 | Locknut, Serrated Flange Hex |

Additional Items Required For Installation

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |

Conflicts

None

Recommended Tools

| Personal & Vehicle Protection | Notes |
| --- | --- |
| Blanket/Seat Protection |  |
| Special Tools | Notes |
| Installation Tools | Notes |
| Ratchet |  |
| Torque Wrench | 0-30 ft-lbs (0-40 N-m) |
| Extension | 9” |
| Sockets | 10mm & 12mm |
| Universal Joint |  |
| Hex Socket | 3mm, Ball Drive |
| Utility Knife |  |
| Panel Removal Tool |  |
| Tape | Removable (i.e. masking) |
| Special Chemicals | Notes |

General Applicability

All iA models

Recommended Sequence of Application

| Item # | Accessory |
| --- | --- |
| 1 |  |
| 2 |  |

*Mandatory

Vehicle Service Parts (may be required for reassembly)

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

Legend

STOP: Damage to the vehicle may occur.  Do not proceed until process has been complied with.

OPERATOR SAFETY: Use caution to avoid risk of injury.

CAUTION: A process that must be carefully observed in order to reduce the risk of damage to the accessory/vehicle and to ensure a quality installation.

TOOLS & EQUIPMENT: Used in Figures calls out the specific tools and equipment recommended for this process.

REVISION MARK: This mark highlights a change in installation with respect to previous issue.

SAFETY TORQUE: This mark indicates that torque is related to safety.

Care must be taken when installing this accessory to ensure damage does not occur to the vehicle.  The installation of this accessory should follow approved guidelines to ensure a quality installation.

These guidelines can be found in the "Accessory Installation Practices" document.

This document covers such items as:-

- Vehicle Protection (use of covers and blankets, cleaning chemicals, etc.).

- Safety (eye protection, rechecking torque procedure, etc.).

- Vehicle Disassembly/Reassembly (panel removal, part storage, etc.).

- Electrical Component Disassembly/Reassembly (battery disconnection, connector removal, etc.).

Please see your Toyota dealer for a copy of this document.

Disassemble the Center Console.

Fig. 1-1

Firmly grip and pull up the storage bin to remove it (Fig. 1-1).

Turn the storage cup over to expose the white clips, two on the back and one on each side.

Fig. 1-2

Panel removal tool

Use a panel removal tool to remove the two white clips on the back of the storage cup.  Insert the tool under the clip face and press down into the groove (Fig. 1-2).

- NOTE: Keep the clips for reuse.

Remove the side clips from the storage cup.

Fig. 1-3

Press in with the index finger and thumb on the outside face of the white clip (Fig. 1-3).

Fig. 1-4

Panel removal tool

While pressing on the face of the clip, use a panel removal tool to catch the inside of the clip and slide it off (Fig. 1-4).

- NOTE: The clips will break if subjected to extreme bending.

- NOTE: Keep the clips for reuse.

Fig. 1-5

Lift up from the corner edges of the gear selector bezel around the shift lever and remove it (Fig. 1-5).

Fig. 1-6

Use both hands to remove the gear selector bezel from the tab inserts (Fig. 1-6).

Fig. 1-7

Firmly grip underneath the rear side of the cup holder panel and pull up to remove it (Fig. 1-7).  Release the part and guide it through the parking brake lever cutout.

Fig. 1-8

Do not disconnect the panel, set it aside safely against the passenger seat on the floor (Fig. 1-8).  Protect the part from damage.

Fig. 1-9

Utility knife

CORRECT 

INCORRECT 

Use a knife to cut out the tabs in the inner main console (Fig. 1-9).

- NOTE: Cut on the outer edge of the tab (Fig. 1-9).

Fig. 1-10

Utility knife

SCORE

SCORE

SCORE

Use a knife to cut a square pattern and score towards the outer walls (Fig. 1-10).  Push down on the cut plastic and remove it to expose the 10mm hex nuts.

Fig. 1-11

10mm & 12mm sockets, ratchet & extension

10mm

12mm

10mm

Remove the 10mm hex nuts (Fig. 1-11).  Keep them for reuse.

Remove the 12mm bolt (Fig. 1-11).  Keep it for reuse.

##### Install the Armrest Bracket.

Fig. 2-1

Insert the armrest bracket legs into the slots (Fig. 2-1).

Fig. 2-2

NOTE: Use caution when installing the bracket, as it sits in close proximity to the parking brake cable (Fig. 2-2).

Fig. 2-3

10mm socket, torque wrench & extension

10mm

Reinstall the two 10mm hex nuts removed in Step 1(k) (Fig. 2-3).

- Torque: 8 ft-lbs (10.8 N-m)

Fig. 2-4

12mm socket, universal joint, extension & torque wrench

12mm

Reinstall the 12mm bolt removed in Step 1(l) (Fig. 2-4).

- Torque: 29 ft-lbs (39.3 N-m)

- NOTE: The bracket leg hides the bolt head.  A universal joint is required along with a short 12mm socket.

##### Install the Lower Base.

Fig. 3-1

Replace the cup holder panel removed in Step 1(g) (Fig. 3-1).  Ensure all pieces are securely fastened in place.

Fig. 3-2

Replace the gear selector bezel removed in Step 1(f) (Fig. 3-2).  Ensure the tabs enter the slots and is securely fastened in place.

Fig. 3-3

Install the white clips removed in Steps 1(c) and 1(d) to the lower base (Fig. 3-3).

- NOTE: The clips should snap into place with an audible sound.

Fig. 3-4

Insert the lower base over the armrest bracket and push it down firmly to snap all four tabs into the wall openings (Fig. 3-4).

Test the lower base fit by slightly wiggling it to ensure it’s been properly installed.

##### Install the Upper Adapter.

Fig. 4-1

Insert the upper adapter onto the base lower (Fig. 4-1).

Fig. 4-2

3mm ball-drive socket & torque wrench

Use a 3mm ball-drive hex socket to install the four screws (Fig. 4-2).

- Torque: 7 ft-lbs (9.5 N-m)

Fig. 4-3

Insert the driver side inner shield and driver side outer bushing into the fitted adapter space (Fig. 4-3).  The textured sides should be visible on the outside.

Fig. 4-4

Insert the passenger side inner shield and passenger side outer bushing into the fitted adapter space (Fig. 4-4).  The textured sides should be visible on the outside.

Fig. 4-5

Affix removable tape to both the passenger side and driver side shields (Fig. 4-5).

##### Install the Armrest.

Fig. 5-1

Insert the armrest into the adapter (Fig. 5-1).  Position it so the holes are aligned for the carriage bolt.

Fig. 5-2

Insert the carriage bolt with the head of the bolt on the passenger side, through the shields, adapter, and armrest, until it reaches the other side (Fig. 5-2).

- NOTE: Ensure that the square neck locates into the square hole in the upper adapter on the passenger side.

Fig. 5-3

12mm socket & torque wrench

Use a 12mm socket to tighten the serrated-flange, hex locknut onto the threaded side of the bolt (Fig. 5-3).

- Torque: 3 ft-lbs (4 N-m)

Fig. 5-4

Install the caps (right & left) into the adapter, covering the bolt and hex locknut openings (Fig. 5-4).

Fig. 5-5

Lift the armrest and remove the two strips of tape from the inner shields (Fig. 5-5).

Accessory Function Checks

Vehicle Function Checks

Vehicle Appearance Check

After accessory installation and removal of protective cover(s), perform a visual inspection.

Ensure no damage (including scuffs and scratches) was caused during the installation process.

(For PPO installations, refer to TMS Accessory Quality Shipping Standard.)