Part Number:	PT747-12160

Kit Contents

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 | 1 | Graphic, Lower Body RH |
| 2 | 1 | Graphic, Lower Body LH |
| 3 |  |  |

Hardware Bag Contents

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |

Additional Items Required For Installation

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

Conflicts

None

Care must be taken when installing this accessory to ensure damage does not occur to the vehicle.  The installation of this accessory should follow approved guidelines to ensure a quality installation.

These guidelines can be found in the "Accessory

Installation Practices" document.

This document covers such items as:-

• Vehicle Protection (use of covers and blankets, cleaning chemicals, etc.).

• Safety (eye protection, rechecking torque procedure, etc.).

• Vehicle Disassembly/Reassembly (panel removal, part storage, etc.).

• Electrical Component Disassembly/Reassembly (battery disconnection, connector removal, etc.).

Please see your Scion dealer for a copy of this document.

Recommended Tools

| Personal & Vehicle Protection | Notes |
| --- | --- |
| None |  |
| Special Tools | Notes |
| Squeegee | 4" (3M™Gold) or 4" (3M™Silver) |
| Low Friction Sleeve | 3M SA-1 (OPTIONAL) |
| Surface Thermometer | Infrared Non-contact |
| Heat Gun | Master Heat Gun HG-301A or equivalent |
| Installation Tools | Notes |
| Lint Free Rags |  |
| Spray Bottle | For wetting agent |
| Special Chemicals | Notes |
| Cleaner | VDC approved cleaner and cleaning method |
| Wetting Agent Note: Solution depends upon ambient temperature of the part and the vehicle surface. If it is difficult to remove bubbles with a squeegee, add one more ml of shampoo. | Johnson’s Baby Shampoo mixed in one liter of water: At 65˚F add 2 ml. shampoo At 70˚F add 3 ml. shampoo At 80˚F add 3.5 ml shampoo At 90˚F add 4 ml. shampoo |

General Applicability

All iM models

Recommended Sequence of Application

| Item # | Accessory |
| --- | --- |
| 1 | Body Side Molding |
| 2 | Lower Body Graphic |

*Mandatory

Vehicle Service Parts (may be required for reassembly)

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |

Legend

STOP: Damage to the vehicle may occur.  Do not proceed until process has been complied with.

OPERATOR SAFETY: Use caution to avoid risk of injury.

CAUTION: A process that must be carefully observed in order to reduce the risk of damage to the accessory/vehicle and to ensure a quality installation.

TOOLS & EQUIPMENT: Used in Figures calls out the specific tools and equipment recommended for this process.

REVISION MARK: This mark highlights a change in installation with respect to previous issue.

SAFETY TORQUE: This mark indicates that torque is related to safety.

Storage

90˚F

OK

65˚F

Installation Temperature

Cleaner

Squeegee

Sleeve (Optional)

Lint Free

Lower Body Graphic LH

3

2

1

DRY

b

CLEAN

a

Lower Body Graphic RH

SQUEEGEE

Do not remove pre-mask paper until after rear door graphic is applied

1

2

ALIGN

Align bottom edge of pre-mask paper to bottom of door panel

ALIGN

Align notch to door edge

5

6

4

7

9

SQUEEGEE

= OPTIONAL

SLEEVE

8

10

180°

PULL

12

DRY

Repeat steps 1 – 12 on driver side

11

SQUEEGEE

Accessory Function Checks

Proper adhesion – visual

Proper adhesion – physical

Proper alignment – cross-car

Vehicle Function Checks

Vehicle Appearance Check

After accessory installation and removal of protective cover(s), perform a visual inspection.

Bubbles, loose and lifting edges

With light pressure, rub hand over top of part and if crackling sound occurs, re- squeegee. If bubbles cannot be worked out to the nearest edge, replace part.

Part must have a uniform gap around door edge +/- 1.0 mm.

Ensure no damage (including scuffs and scratches) was caused during the installation process.

(For PPO installations, refer to TMS Accessory Quality Shipping Standard.)