Part Number:	PU322-42165-WH

Kit Contents

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 | 1 | Module Trailer Harness |
| 2 | 1 | 4-Flat Trailer Harness |
| 3 | 1 | Mounting Bracket: 4-Flat |
| 4 | 2 | Screw #10-24 |
| 5 | 2 | Nut/Washer #10-24 |
| 6 | 45 | Wire Tie: 8” |
| 7 | 1 | Wire Tie: Stud Mount (M6) |
| 8 | 4 | Wire Tie: 14” |
| 9 | 1 | Wire Tie: Push Mount Clip (Grey) |
| 10 | 6 | Foam Pad |
| 11 | 1 | Fuse Location Card |

Hardware Bag Contents

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

Additional Items Required For Installation

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 | 1 | Tow Hitch |
| 2 |  |  |
| 3 |  |  |

Conflicts

Gas (non-hybrid) vehicles & SE models

General Applicability

2016 and later RAV4 Hybrid models except SE grade

Recommended Sequence of Application

| Item # | Accessory |
| --- | --- |
| 1 | Module Trailer Harness |
| 2 | Towing Hitch |
| 3 | 4-Flat Trailer Harness |

*Mandatory

Vehicle Service Parts (may be required for reassembly)

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

Recommended Tools

| Personal & Vehicle Protection | Notes |
| --- | --- |
| Safety Glasses |  |
| Safety Shoes |  |
| Blankets |  |
| Special Tools | Notes |
| Installation Tools | Notes |
| Wrench | 10mm |
| Ratchet | ⅜” Drive |
| Extension | ⅜” Drive, Medium |
| Socket | 10mm, 14 mm ⅜” Drive |
| Screwdriver | Phillips #2 |
| Drill |  |
| Drill Bit | 7.5mm |
| Side Cutter |  |
| Plastic Trim Removal Tool |  |
| Tape Measure |  |
| Flashlight |  |
| Torque Wrench | 5.4 N-m (48 in-lbf) 8.4 N-m (74 in-lbf) 42 N-m (371 in-lbf) |
| Special Chemicals | Notes |
| Cleaner |  |

Legend

STOP: Damage to the vehicle may occur.  Do not proceed until process has been complied with.

OPERATOR SAFETY: Use caution to avoid risk of injury.

CAUTION: A process that must be carefully observed in order to reduce the risk of damage to the accessory/vehicle and to ensure a quality installation.

TOOLS & EQUIPMENT: Used in Figures calls out the specific tools and equipment recommended for this process.

REVISION MARK: This mark highlights a change in installation with respect to previous issue.

SAFETY TORQUE: This mark indicates that torque is related to safety.

Care must be taken when installing this accessory to ensure damage does not occur to the vehicle.  The installation of this accessory should follow approved guidelines to ensure a quality installation.

These guidelines can be found in the "Accessory Installation Practices" document.

This document covers such items as:-

- Vehicle Protection (use of covers and blankets, cleaning chemicals, etc.).

- Safety (eye protection, rechecking torque procedure, etc.).

- Vehicle Disassembly/Reassembly (panel removal, part storage, etc.).

- Electrical Component Disassembly/Reassembly (battery disconnection, connector removal, etc.).

Please see your Toyota dealer for a copy of this document.

Disassemble the Vehicle.

Place the transmission in Park and set the parking brake.

Fig. 1-1

Deck board

Remove the deck board (Fig. 1-1).

Fig. 1-2

No. 3 deck board

asdf

asdf

Trim removal tool

Remove the No. 3 deck board (Fig. 1-2).

asdf

Clip x 2

Fig. 1-3

Battery deck trim cover

asdf

asdf

Trim removal tool

asdf

asdf

asdf

Remove the battery deck trim cover (Fig. 1-3).

asdf

Clip x 5

Fig. 1-4

10mm wrench or socket & ratchet

Disconnect the negative (-) battery terminal from the battery (Fig. 1-4).

- NOTE: Isolate the negative battery terminal.

Fig. 1-5

No. 2 tool box

asdf

Trim removal tool

asdf

asdf

asdf

Remove the No. 2 tool box (Fig. 1-5).

asdf

Clip x 2

asdf

Push pin x 2

Fig. 1-6

Rear floor finish plate

asdf

asdf

Trim removal tool

asdf

asdf

Remove the rear floor finish plate (Fig. 1-6).

asdf

Clip x 4

Fig. 1-7

Rear floor mat support side plate

asdf

asdf

Trim removal tool, #2 Phillips screwdriver

asdf

asdf

asdf

asdf

asdf

Lower the rear seats and remove the rear floor mat support side plate (Fig. 1-7).

asdf

Push pin x 7

- Screw x 4

Fig. 1-8

Rear seat cushion

asdf

asdf

Raise the rear seats and remove the rear seat cushion by lifting the front to release the clips (Fig. 1-8).

asdf

Clip x 2

Fig. 1-9

Rear door scuff plate

asdf

asdf

Trim removal tool

Remove the LH and RH rear door scuff plate (Fig. 1-9).  LH side shown, repeat on the RH side.

asdf

Clip x 2

- Claw x 6

Fig. 1-11

asdf

Fig. 1-10

Rear seat recliner release handle

asdf

Remove the LH and RH rear seat recliner release handle (Fig. 1-10 & Fig. 1-11).  LH side shown, repeat on the RH side.

asdf

Clip x 1

Fig. 1-12

Battery service cover

asdf

Trim removal tool

asdf

asdf

asdf

Remove the LH and RH battery service cover (Fig. 1-12).  LH side shown, repeat on the RH side.

asdf

Clip x 2

asdf

Push pin x 7

Fig. 1-13

Lower the rear seat backs and partially remove the LH and RH rear door weather strip (Fig. 1-13).  LH side shown, repeat on the RH side.

Fig. 1-15

Fig. 1-14

Wrap the LH and RH rear seat recliner handles with shop cloths to prevent damage (Fig. 1-14 & Fig. 1-15).  LH side shown, repeat on the RH side.

Fig. 1-18

Disconnect

Fig. 1-17

Fig. 1-16

Deck trim side panel

asdf

asdf

Trim removal tool, #2 Phillips screwdriver, 10mm & 14mm sockets, ratchet, extension

asdf

asdf

asdf

asdf

asdf

asdf

asdf

asdf

Fig. 1-18

Fig. 1-17

Remove the LH and RH deck trim side panel (Fig. 1-16 to Fig. 1-18).  Disconnect any connectors.  LH side shown, repeat on the RH side.

asdf

Clip x 10

- Screw x 1

- Bolt x 4

Fig. 1-19

Seat belt anchor cover cap

Trim removal tool

Remove the LH and RH seat belt anchor cover cap (Fig. 1-19).  LH side shown, repeat on the RH side.

- Claw x 2

Fig. 1-20

14mm socket, ratchet extension

Remove the LH and RH rear seat belt anchor (Fig. 1-20).  LH side shown, repeat on the RH side.

- Bolt x 1

Fig. 1-21

Roof side garnish

asdf

asdf

Trim removal tool

asdf

asdf

asdf

asdf

asdf

asdf

asdf

Remove the LH and RH roof side garnish (Fig. 1-21).  LH side shown, repeat on the RH side.

asdf

Clip x 9

##### Install the Trailer Harness Module.

Fig. 2-1

Clean the LH rear wheel house area shown in Fig. 2-1 with a locally approved cleaner.

Peel off the tape backing strips from the trailer harness module.

Fig. 2-2

Tape measure

60 mm

Align

Use the reference dimension shown in Fig. 2-2 and apply pressure to secure the trailer harness module to the LH rear wheel house area.

- NOTE: Be sure to leave sufficient clearance to the module for the LH deck trim side panel to be reinstalled.

##### Install the Trailer Wire Harness.

B

D

C

Fig. 3-2

Fig. 3-1

A

Route the trailer wire harness (Fig. 3-1 to Fig. 3-2).

Route the white, brown, yellow & green and green wires (A) towards the tow hitch.

Route the white, brown and yellow wires with a ground ring (B) towards the LH rear combination lamp.

Route the red and green wires (C - Longest) towards RH rear combination lamp.

Route the black wire with the fuse (D) towards the battery.

Fig. 3-3

1

2

4

5

6

7

3

B

C

Secure the trailer wire harness (B and C) to the vehicle harness with 8” wire ties 1 and 2 (Fig. 3-3).

Fig. 3-5

3

4

5

7

6

B

Fig. 3-4

1

2

3

B

C

Secure the trailer wire harness (B) to the vehicle harness with 8” wire ties 3 to 7 (Fig. 3-3 to Fig. 3-5).

10mm socket, ratchet, torque wrench, extension

Fig. 3-6

B

Secure the trailer wire harness (B) ground terminal to the vehicle with the original ground bolt (Fig. 3-6).

- Bolt x 1

- Torque: 8.4 N-m (74 in-lbf)

Fig. 3-9

11

12

13

14

15

16

B

Fig. 3-8

5

6

7

8

9

10

11

B

Fig. 3-7

7

6

8

10

9

11

12

13

14

15

16

B

Secure the trailer wire harness (B) to the vehicle harness with 8” wire ties 8 to 16 (Fig. 3-7 to Fig. 3-9).

The #9 wire tie latch must be pointed down, away from the ground wire.

The #15 wire tie latch must be installed away from the OE splice junction (Fig. 3-9 inset).

- NOTE: Do not secure the trailer wire harness (B) to the water hose, which must be free for proper operation.

Fig. 3-10

B

Trailer wire harness (B) T-connection

Vehicle harness 4-pin male connector

Vehicle LH RR combination lamp 4-pin female connector

Plug the trailer wire harness (B) T-connection 4-pin male connector (with white, brown, yellow and blue wires) to the 4-pin female connector from the vehicle LH rear combination lamp (Fig. 3-10).

Plug the trailer wire harness (B) T-connection 4-pin female connector to the 4-pin male connector from the vehicle harness (Fig. 3-10).

Fig. 3-11

4-pin connector to vehicle harness

Trailer wire harness junction block

Wrap the plastic junction block of the trailer wire harness (B) and the loose vehicle harness 4-pin connector with foam pads (Fig. 3-11).

Fig. 3-12

17

15

14

13

B

Bundle the plastic junction block of the trailer wire harness (B) and the loose vehicle harness 4-pin connector with 8” wire tie 17 (Fig. 3-12).

Fig. 3-14

3

18

19

20

21

22

23

C

B

Fig. 3-13

4

3

18

19

20

21

22

23

24

25

26

27

C

B

Secure the trailer wire harness (C) to the vehicle harness with 8” wire ties 18 to 23 (Fig. 3-13 to Fig. 3-14).

33

30

31

32

C

Fig. 3-17

Fig. 3-16

28

29

30

31

27

C

Fig. 3-15

28

22

23

24

25

26

27

C

Secure the trailer wire harness (C) to the vehicle harness with 8” wire ties 22 to 33 (Fig. 3-15 to Fig. 3-17).

- NOTE: The #28-33 wire tie latches must be positioned away from all drain hoses along the wire path.

Fig. 3-18

C

Trailer wire harness (C) T-connection

Vehicle harness 4-pin male connector

Vehicle RH RR combination lamp 4-pin female connector

33

Plug the trailer wire harness (C) T-connection 4-pin male connector (with red, green, white and blue wires) to the 4-pin female connector from the vehicle RH rear combination lamp (Fig. 3-18).

Plug the trailer wire harness (C) T-connection 4-pin female connector to the 4-pin male connector from the vehicle harness (Fig. 3-18).

Fig. 3-19

4-pin connector to vehicle harness

Trailer wire harness junction block

Wrap the plastic junction block of the trailer wire harness (C) and the loose vehicle harness 4-pin connector with foam pads (Fig. 3-19).

33

34

C

Fig. 3-20

Bundle the plastic junction block of the trailer wire harness (C) and the loose vehicle harness 4-pin connector with 8” wire tie 34 (Fig. 3-20).

Fig. 3-21

D

Remove 15A fuse

Remove the positive (+) battery cover and 15A fuse from the trailer wire harness (D).  Connect the trailer wire harness (D) to the B+ battery terminal (Fig. 3-21).

##### Nut x 1

- Torque: 5.4 N-m (48 in-lbf)

Fig. 3-22

1

38

37

36

35

C

B

D

Secure the trailer wire harness (D) to the vehicle harness with 8” wire ties 35 to 38 (Fig. 3-22).

Fig. 3-23

D

15A fuse

Reinstall the 15A fuse to the trailer wire harness (D) (Fig. 3-23).

Fig. 3-24

Install the tow hitch before proceeding (Fig. 3-24).  Refer to the tow hitch installation instructions.

Fig. 3-25

Hole plug

Remove the vehicle hole plug located below the LH rear combination lamp (Fig. 3-25).

Fig. 3-27

4-flat trailer harness

Fig. 3-26

4-flat trailer harness

Route the 4-flat trailer wire harness through the vehicle hole (Fig. 3-26 to Fig. 3-27).

Fig. 3-28

4-flat trailer harness grommet

Ensure that the 4-flat trailer wire harness grommet is fully seated to the vehicle body (Fig. 3-28).

Fig. 3-29

A

4-flat trailer harness

Plug the 4-flat trailer wire harness to the trailer wire harness (A) (Fig. 3-29).

Fig. 3-30

A

4-flat trailer harness

39

Wrap the connector with a foam pad and secure it to the vehicle harness with wire tie 39 (Fig. 3-30).

Fig. 3-31

44

40

41

42

43

M6 stud mount clip

4-flat trailer harness

Route and secure the 4-flat trailer wire harness to the tow hitch with the stud mount clip and 8” wire tie 40 and 14” wire ties 41 to 44 (Fig. 3-31).

##### M6 stud mount clip x 1

Cut off the excess from the wire ties.

##### Modify the LH Undercover.

Fig. 4-1

15mm

15mm

Drill the LH undercover with a 7.5 mm drill bit in the location shown in Fig. 4-1).

Reinstall the modified rear bumper fascia and undercovers before proceeding.  Refer to the tow hitch installation instructions.

##### Mount the Trailer Harness Connector.

Fig. 5-2

Fig. 5-1

4-flat trailer harness

L-shape bracket

Assemble the 4-flat trailer wire harness to the L-shape bracket and attach it to the tow hitch (Fig. 5-1 to Fig. 5-2).

##### Screw x 2

##### Nut/Washer x 2

Fig. 5-3

Push mount clip

4-flat trailer harness

Secure the 4-flat trailer wire harness to the LH undercover with the push mount clip (Fig. 5-3).

##### Push mount clip x 1

Cut off the excess from the push mount clip.

##### Reassemble the Vehicle.

Reinstall the components removed in Step 1 in reverse order.

Verify the trim panels fit together properly with no uneven gaps between them.

The bolt torque for the seat belt anchor is 42 N-m (31 in-lbf).

- Torque: 42 N-m (31 in-lbf)

The bolt torque for the negative battery terminal is 5.4 N-m (48 ft-lbf).

- Torque: 5.4 N-m (48 in-lbf)

Place the installation instructions, fuse location card, and owner’s manual in the glove box for future reference.

Perform the functional check for the trailer wire harness and the post installation check for the vehicle.  Refer to the attached checklist.

- WARNING: Overloading the circuit can cause a fire.  DO NOT exceed the load of the towing manufacturer rating or:

- Maximum stop/turn light:  2 bulbs per side (total 4.2 amps)

- Maximum tail lights:  7.5 amps

Read the vehicle owner’s manual and instruction sheet for additional information.

Clean up and remove any trash.

Accessory Function Checks

Right turn signal

Left turn signal

Brake

Tail light

Tail light & brake

Left turn & brake

Right turn & brake

Vehicle Function Checks

Individual vehicle light functions

Trunk light

Warning lights

Front wipers & washers

Circuit 1 flashes

Circuit 2 flashes

Circuit 1 & 2 activate

Circuit 3 activates

Circuit 1, 2 & 3 activate

Circuit 1 activates & 2 flashes

Circuit 1 flashes & 2 activates

Verify that the corresponding vehicle lights activate.  If light does not activate then check the connection.

Functions normally

No warning lights illuminated in the combination meter

Operate the front wiper while spraying washer fluid onto the windshield glass. Make sure that the front wiper functions properly and the wiper does not come into contact with the vehicle body.

Lift each front wiper blade twice after the front wipers stop and check the wiper set position.

Vehicle Appearance Check

After accessory installation and removal of protective cover(s), perform a visual inspection.

Failure Mode

- No power to the trailer light functions

Corrosion on the 4-flat plug

Ensure no damage (including scuffs and scratches) was caused during the installation process.

(For PPO installations, refer to TMS Accessory Quality Shipping Standard.)

Troubleshooting Steps

- Check the trailer wiring harness fuse per the owner’s manual fuse location card.

- Check the 4-flat connection between the trailer and the vehicle to ensure the connection is fully engaged.

- Turn all functions off and remove the key for 10 seconds and retest the functions with the trailer attached.

- Turn all functions off and remove the key for 10 seconds with the trailer disengaged.  Use a multi-meter to check the function output at the 4-flat.  If all functions work, check the trailer for a potential short or overload described in the owner’s manual fuse location card.

Clean the connection and add Dielectric Grease (sold separately) to the terminal pins.