Part Number:	PU060-12114-P1

PU060-12115-P1

Kit Contents

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 | 1 | Front Passenger Side Splashguard |
| 2 | 1 | Front Driver Side Splashguard |
| 3 | 1 | Rear Passenger Side Splashguard |
| 4 | 1 | Rear Driver Side Splashguard |
| 5 | 1 | Hardware Kit |
| 6 | 1 | Instruction Sheet (Electronic Copy) |

Hardware Bag Contents

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 | 2 | M5 x 20 mm Screws |
| 2 | 4 | #14 X 19mm Screws |
| 3 | 6 | M5 Spring Clips (small) |
| 4 | 6 | M5 x 15mm Screws |
| 5 | 4 | #14 x 19mm Hex Screws |
| 6 | 2 | M5 Spring Clips (large) |

Additional Items Required For Installation

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

Conflicts

Ground effects

Recommended Tools

| Personal & Vehicle Protection | Notes |
| --- | --- |
| None |  |
| Special Tools | Notes |
| None |  |
| Installation Tools | Notes |
| Screwdriver | Phillips #2, #3 |
| Socket & Driver | 10mm (3/8”) |
| Clean Cloth/Rag |  |
| Drill | 90 ° / 45 ° |
| Drill Bit | 6mm (1/4”) |
| Torque Wrench | 0-100 ft-lbf |
| Special Chemicals | Notes |
| Cleaner | VDC Approved Cleaner |

General Applicability

All Models

Recommended Sequence of Application

| Item # | Accessory |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |

*Mandatory

Vehicle Service Parts (may be required for reassembly)

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

Legend

STOP: Damage to the vehicle may occur.  Do not proceed until process has been complied with.

OPERATOR SAFETY: Use caution to avoid risk of injury.

CAUTION: A process that must be carefully observed in order to reduce the risk of damage to the accessory/vehicle and to ensure a quality installation.

TOOLS & EQUIPMENT: Used in Figures calls out the specific tools and equipment recommended for this process.

REVISION MARK: This mark highlights a change in installation with respect to previous issue.

SAFETY TORQUE: This mark indicates that torque is related to safety.

Care must be taken when installing this accessory to ensure damage does not occur to the vehicle.  The installation of this accessory should follow approved guidelines to ensure a quality installation.

These guidelines can be found in the "Accessory Installation Practices" document.

This document covers such items as:-

- Vehicle Protection (use of covers and blankets, cleaning chemicals, etc.).

- Safety (eye protection, rechecking torque procedure, etc.).

- Vehicle Disassembly/Reassembly (panel removal, part storage, etc.).

- Electrical Component Disassembly/Reassembly (battery disconnection, connector removal, etc.).

Please see your Toyota dealer for a copy of this document.

Fig. 1-1

Area to be cleaned

Clean Front Mounting Area

Clean and degrease front body panel where splash guard is to be located with a clean cloth and VDC approved cleaner (Fig. 1-1).

- Installation of Front Splash Guard

#3 Phillips Screwdriver

Supplied

#14 Screws

A

B

Splash guard

Fig. 2-1

(a) 	Mount splash guard on body panel and install supplied #14 screw in location B while holding part up and into body panel, then tighten        (Fig. 2-1).

- Do Not Over Tighten.

  - Install supplied #14 screw in location A while holding part up and into body panel, then tighten (Fig. 2-1).                Do Not Over Tighten.

Supplied #14 Hex screw

C

10mm Socket/Driver
#3 Phillips Screwdriver

Fig. 2-2

Install supplied #14 hex screw in location C while holding part up and into body panel, then tighten        (Fig. 2-2).

- Do Not Over Tighten.

Repeat procedure for other side.

##### Clean Rear Mounting Area and Remove Wheel

NOTE: REMOVAL AND REINSTALLATION OF REAR WHEELS IS REQUIRED TO DRILL PANEL EDGE ATTACHMENT HOLES UNLESS A SIDE-ANGLE DRILL IS USED.

        - Clean and degrease rear bumper panel where splash guard is to be located with a clean cloth and VDC approved cleaner.

        - If straight and accurate holes cannot be drilled, support the vehicle and remove the rear wheel.  Follow the Owner’s Manual procedure.

Fig. 4-1

Drill / 6mm Drill Bit

B

6mm Bit

A

Drill

Fig. 4-2

C

Supplied Spring Clips

(small)

B

A

Supplied Spring Clip

(large)

Hole Preparation and Clips

Drill 6mm (1/4”) holes in panel edge at dimple locations A and B (Fig. 4-1).

Install supplied spring clips (small) over panel edge holes locations A and B with turned up edge on inside of panel (Fig. 4-2).

Install supplied spring clip (large) over panel edge hole location C with turned up edge on inside of panel (Fig. 4-2).

#2 Phillips Screwdriver

B

Splash guard

Supplied

M5 x 15mm

Screws

A

Fig. 5-1

(a)

Installation Of Rear Splash Guard

- Mount splash guard and install M5 screw 15mm in location B then A while holding part up and into bumper panel and tighten (Fig. 5-1).

#3 Phillips Screwdriver

10mm Socket/Driver

Supplied

#14 x 19mm

Hex Screw

Supplied

M5 x 20mm screw

C

D

Fig. 5-2

  - Install supplied M5 screw 20 mm in location C while holding part up and into bumper panel and tighten         (Fig. 5-2).

Fig. 5-3

Install supplied #14 hex screw in location D while holding part up and into bumper panel and tighten          (Fig. 5-2).

Using part as a template hold tight to panel and drill 6mm ( 1/4 “) hole through panel edge and liner at location E (Fig. 5-3).

Fig. 5-4

Install supplied spring clip (small) over panel edge hole with turned up edge on inside of panel and between liner at location E (Fig. 5-4).

Fig. 5-5

Install supplied M5 screw 15mm in location E and tighten (Fig. 5-5).

If the wheel was removed Step 3 (b), reinstall it now and torque the lug nuts to 103 N∙m (76 ft∙lbf).

##### Torque: 103 N∙m (76 ft∙lbf)

Repeat procedure for other side.

Accessory Function Checks

Gaps between mudguards and body side

Inspect torques

Vehicle Appearance Check

After accessory installation and removal of protective cover(s), perform a visual inspection.

Excessive gaps greater than 1.0 mm.

If the wheels were removed, torque the lug nuts to 103 N•m (76 ft•lbf).

Ensure no damage (including scuffs and scratches) was caused during the installation process.

(For PPO installations, refer to TMS Accessory Quality Shipping Standard.)