Part Number:	PU060-08015-P1

Kit Contents

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 | 1 | Front Passenger Side Mudguard |
| 2 | 1 | Front Driver Side Mudguard |
| 3 | 1 | Rear Passenger Side Mudguard |
| 4 | 1 | Rear Driver Side Mudguard |
| 5 | 2 | Hardware Kit |
| 6 | 1 | Instruction Sheet (Electronic Copy) |

Hardware Bag Contents

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 | 6 | #10 x 15 mm Screws (short) |
| 2 | 4 | #10 x 19 mm Screws (long) |
| 3 | 6 | #10 Spring Clips (short) |
| 4 | 2 | #10 Spring Clips (long) |

Additional Items Required For Installation

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

Conflicts

Recommended Tools

| Personal & Vehicle Protection | Notes |
| --- | --- |
| None |  |
| Special Tools | Notes |
| None |  |
| Installation Tools | Notes |
| Screwdriver | Phillips #2 & 3, Flat Blade |
| Drill | Standard/Right Angle |
| Drill Bits | 2.5 mm ( 3/32” ) |
|  | 6 mm ( 1/4” ) |
| Clean Cloth/Rag |  |
| Special Chemicals | Notes |
| Soap & Water Mixture |  |

General Applicability

Recommended Sequence of Application

| Item # | Accessory |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |

*Mandatory

Vehicle Service Parts (may be required for reassembly)

| Item # | Quantity Reqd. | Description |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

Legend

STOP: Damage to the vehicle may occur.  Do not proceed until process has been complied with.

OPERATOR SAFETY: Use caution to avoid risk of injury.

CAUTION: A process that must be carefully observed in order to reduce the risk of damage to the accessory/vehicle and to ensure a quality installation.

TOOLS & EQUIPMENT: Used in Figures calls out the specific tools and equipment recommended for this process.

REVISION MARK: This mark highlights a change in installation with respect to previous issue.

SAFETY TORQUE: This mark indicates that torque is related to safety.

Care must be taken when installing this accessory to ensure damage does not occur to the vehicle.  The installation of this accessory should follow approved guidelines to ensure a quality installation.

These guidelines can be found in the "Accessory Installation Practices" document.

This document covers such items as:-

- Vehicle Protection (use of covers and blankets, cleaning chemicals, etc.).

- Safety (eye protection, rechecking torque procedure, etc.).

- Vehicle Disassembly/Reassembly (panel removal, part storage, etc.).

- Electrical Component Disassembly/Reassembly (battery disconnection, connector removal, etc.).

Please see your Toyota dealer for a copy of this document.

Prepare to Mount the Front Mudguards.

Clean and degrease the front body panel on the driver’s side where the mudguard will be located with a clean cloth and VDC approved cleaner or soap/water mixture.

Fig. 1-1

#3 Phillips screwdriver

Existing Screws

A

B

Remove the exisiting screws, locations A and B (Fig. 1-1).  DO NOT DISCARD THEM.

Install the Front Mudguards.

Hold the part up and into the body panel.

Fig. 2-1

#3 Phillips screwdriver

Factory Screws

A

B

Mudguard

Reinstall the OE screws in locations A and B (Fig. 2-1).

Fig. 2-2

#2 Phillips screwdriver, drill motor & 2.5mm drill

C

Supplied 19mm screw (long)

Hold the part in position and drill a 2.5mm (3/32”) pilot hole in location C (Fig. 2-2).

Hold the part into the body panel and install a supplied 19mm (long) self-tapping screw into location C (Fig. 2-2).

Repeat Steps 1 and 2 on the passenger’s side.

Prepare to Mount the Rear Mudguards.

Clean and degrease the rear body panel on the driver’s side where the mudguard will be located with a clean cloth and VDC approved cleaner or soap/water mixture.

Fig. 3-1

Flat blade screwdriver

Existing push pin (discard)

D

Remove the exisiting push pin, location D, and discard it (Fig. 3-1).

Install the Rear Mudguards.

Hold the part up and into the body panel.

Fig. 4-1

Drill motor & 2.5mm drill

6mm drill bit

A

D

Passenger’s side only

Mudguard

Drill motor

B

C

Use the part as a template and drill 6mm (1/4”) holes in the panel edge at locations A, B and C (Fig. 4-1).

- NOTE: For easier accessibility, the wheels can be removed or use a right-angle drill motor.

Passenger’s side only: Hold the part up and into the body panel and drill at location D (Fig. 4-1).

Fig. 4-2

Spring clip (long)

Supplied spring clips (small)

Install the supplied spring clips (small) over the panel edge holes at location A, B and C (Fig. 4-1 & Fig. 4-2).

- NOTE: The turned up edge goes on the inside of the panel.

Pull the rear liner down and install a supplied spring clip (long) over the panel edge hole at location D (Fig. 4-1 & Fig. 4-2).

- NOTE: The turned up edge goes on the inside of the panel.

Driver’s side only:

Fig. 4-3

#2 Phillips screwdriver

Supplied 19mm screw (long)

D

Supplied spring clip (long)

Hold the mudguard up and into the body panel and install the supplied 19mm (long) screw in location D (Fig. 4-3).

Passenger’s side only: Hold the part up and into the body panel and install the supplied 19mm screw (long) and the #10 spring clip (long) as a nut in location D (Fig. 4-3).

Fig. 4-4

#2 Phillips screwdriver

A

Mudguard

B

C

Supplied screws (short)

Hold the mudguard up and into the body panel and install the supplied 15mm (short) screws in locations A, B and C (Fig. 4-4).

Repeat Steps 3 and 4 on the passenger’s side.

Accessory Function Checks

Gap between the mudguard and the body

Vehicle Function Checks

Excessive gaps greater than 1.0 mm

Vehicle Appearance Check

After accessory installation and removal of protective cover(s), perform a visual inspection.

Ensure no damage (including scuffs and scratches) was caused during the installation process.

(For PPO installations, refer to TMS Accessory Quality Shipping Standard.)